Charles W. Davidson College of Engineering

Department of Industrial and Systems Engineering

ISE 140 – Group # 11

Term Project

“Forecasting, Planning, and Scheduling: Transformative Strategies for JJJ Enterprise’s Vikings Division”

Prepared By:

Aditya Nayak

Arul Kumarn Thiagarasu

Kathy Raina

Naveen Kumar K.A.

Prepared For:

Dr. Anil Kumar

May 08, 2024

Table of Contents

1. Executive Summary…………………..………………………………………………………...3

2. Introduction………………...…...…………………….………………………………………...3

2.1 Background….……………………………………………………..……………………...3

2.2 Problem Statement…………………………………….…….…………………………….4

2.3 Objective……………………….…………………………….…………………………....4

3. Methods & Procedures……………………………….…………………………………………4

3.1 Forecasting…………..…………………………………………………………………….5

3.2 Aggregate Planning…………………………..……………………………………………8

3.3 Master Production Scheduling (MPS)...............................................................................10

3.4 Material Requirement Planning (MRP)……………………………..…………………...10

3.5 Capacity Planning…………………………………………...………...…………….…...10

3.6 Simulation and Scheduling………………………………………...…………………….11

4. Results…………….……………………………………………………………..…………….11

4.1 Forecasting……………….………………………….……………………………...……11

4.2 Aggregate Planning…………………………………………..…………………………..13

4.3 Materials Requirement Planning (MRP)...........................................................................14

4.4 Master Production Scheduling (MPS)…………………………………………………...14

4.5 Capacity Planning…………………..………………….…………………...……………14

4.6 Simulation and Scheduling…………………...………………………………………….15

5. Conclusions and Recommendations…………………………………………………………..16

6. References……………………………………………………………………………………..17

7. Appendices…………………………………………………………………………………….18

# 1. Executive Summary

The Vikings Division, operating under JJJ Enterprise, has encountered significant challenges regarding profitability, scheduling delays, and extended cycle times across its diverse product range, impacting revenue and market share. To address these issues, our engineering team conducted a comprehensive investigation and formulated strategies aimed at enhancing revenue, streamlining scheduling, and implementing cost-effective measures tailored to each product within the division. Our approach involved developing optimal forecasts, Bill of Materials (BOM), production schedules, Materials Required Plan/Master Production Schedule, and capacity plans for all product categories. Utilizing a simulation model, we generated visualizations to identify the most efficient methods for minimizing cycle times and improving scheduling across the product spectrum. After thorough analysis, we recommend adopting a level policy for aggregate planning for products 3, 4, and 5, while a chase policy is more suitable for products 1 and 2.

Through meticulous calculations, the total optimal cost for operations and production has been estimated at $1,053,844. Implementation of these strategies and policies is imperative for addressing the challenges confronting Vikings Division. These initiatives are pivotal for the company's future, promising enhanced profitability, streamlined scheduling, and cost-efficient production across its product lines. The proposed measures offer a pathway to overcoming the current obstacles and positioning Vikings Division for sustained success. By embracing these recommendations, the company can achieve greater profitability, operational efficiency, and competitiveness in the market.

# 2. Introduction

2.1 Background

The Vikings Division, operating under JJJ Enterprise, is headquartered in Pleasanton, CA, specializing in the production of various nutritional products, including adult medical nutrition supplements, pediatric infant formulas, and ancillary equipment. Initially, the division enjoyed a competitive advantage with a patented product range, experiencing minimal competition. However, with the expiration of their patent and the increasing demand for nutritional products, numerous competitors entered the market, challenging Vikings' market position. Over the years, the division has encountered significant operational challenges, leading to a decline in profitability and customer satisfaction. Despite experiencing fluctuating demand for their products, the company struggles to meet customer orders promptly, resulting in a history of backordered demands. Furthermore, inefficient production processes contribute to high levels of work-in-process (WIP) inventory and prolonged production cycle times.

These challenges are compounded by factors such as inadequate forecasting methodologies, underutilization of production machinery, inappropriate lot sizes, and the absence of a structured scheduling methodology. As a result, the division operates in an unsustainable environment, jeopardizing its long-term viability. Ms. Jasmine Enterprise, an industrial engineering manager within the company, has been tasked with addressing these pressing issues and restoring operational efficiency. Her efforts are crucial in repairing the division's relationship with customers, optimizing production processes, and ensuring sustainable profitability in the face of intensifying market competition.

# 2.2 Problem Statement

The Vikings Division, a subsidiary of JJJ Enterprise, based in Pleasanton, CA, faces significant challenges in its production of nutritional products. With the expiration of their patent and the emergence of competitors in the market, the company struggles to maintain profitability and meet customer demand efficiently. The demand for their five main products fluctuates, with varying levels of seasonality and week-to-week variability. Despite this, the company has experienced a consistent history of backordered demands, poor cost containment, and high levels of work-in-process (WIP) inventory, leading to prolonged production cycle times. These challenges stem from factors such as inadequate forecasting, improper machine utilization, inappropriate lot sizes, and the absence of a formal scheduling methodology. To address these issues and ensure the division's long-term viability, urgent measures are required to optimize production processes, enhance demand forecasting accuracy, and implement effective cost containment strategies.

# 2.3 Objective

This project aims to increase profits and have a better methodology for the division. The aim of this project is to revitalize the operational efficiency and profitability of the Vikings Division within JJJ Enterprise. By addressing the underlying challenges related to demand forecasting, production planning, and scheduling, the project seeks to optimize production processes, enhance customer satisfaction, and ensure sustainable growth in the increasingly competitive market for nutritional products. Through the implementation of robust forecasting methodologies, efficient resource utilization, and the adoption of formal scheduling practices, the project aims to streamline operations, reduce backorders, minimize work-in-process inventory, and ultimately restore profitability to the division.

# 3. Methods and Procedures

The enhancement process initiated with a strategic overhaul of forecasting techniques. Dedicated engineers were assigned to each of the five main products, tasked with developing a tailored forecasting methodology that leverages historical demand data and market analysis. After establishing forecasts, the team explored both level and chase demand strategies to align production with anticipated market demands. Subsequently, attention was turned to Material Requirements Planning (MRP) and Master Production Schedules (MPS). These were meticulously crafted by determining the necessary subassemblies for each product, leading to a coherent weekly production schedule that balances efficiency with demand fulfillment. The final phase involved the deployment of simulation software to refine machine scheduling. Inputs such as machine capacities, processing times, batch sizes, and routing logic were programmed to generate a preliminary simulation model. This model facilitated rigorous experimentation with batch sizing and scheduling adjustments, aiming to optimize throughput while maintaining operational flexibility. This comprehensive approach not only streamlined production processes but also enhanced the division's responsiveness to market fluctuations.

# 3.1 Forecasting

Forecasting predicts how much we'll need to make in the future based on past sales information. There are different ways to do this, like looking at trends, and demand patterns or using smoothing methods, The method we use depends on the historical data collected.

# (a) Product 1:

A comprehensive analysis of Product 1's demand patterns over the past 20 weeks reveals a consistent and steady demand without any discernible trends or seasonal variations (Figure 1.1). To forecast the demand for the next five weeks, the exponential smoothing method with an alpha value of 0.1 was identified as the most suitable, yielding the lowest Mean Absolute Deviation (MAD). Based on this model, the forecasted demand for week 21 is 145 units (refer to Table 1.1), assuming a consistent demand pattern.

# (b) Product 2:

Observations of Product 2's demand pattern indicate a clear seasonal trend recurring every nine weeks (Figure 2.1). Utilizing this seasonal pattern, the forecasted demand for weeks 21 to 25 was derived, incorporating the latest levels, trends, and standardized seasonal indexes (Table 2.1). This approach resulted in a Mean Absolute Deviation (MAD) of 3.06 (Table 2.2), validating the accuracy of the forecasted values.

# (c) Product 3:

Analysis of Product 3's demand pattern reveals a recurring seasonal cycle with a four-week period (Figure 3.1). Various forecasting techniques, including Moving Average, Naïve Method, and exponential smoothing, were evaluated, with the Linear Regression Model producing the most favorable Mean Absolute Deviation (MAD) of 3.29 (Table 3.2). The forecast for weeks 21 to 25 was generated using a combination of seasonality and trending methods, aligning with the observed patterns (Table 3.1).

# (d) Product 4:

Product 4's demand data exhibits a noticeable upward trend (Figure 4.1). To forecast the demand for weeks 21 to 25, the linear regression approach was employed, utilizing parameters α = 0.2 and β = 0.4 (Table 4.1). The forecasted values were derived based on the intercept as the base forecast (Ft) and the slope as the trend value (Tt), focusing on the most recent weeks to ensure accuracy.

# (e) Product 5:

Analysis of Product 5's demand data reveals no significant seasonality or trends (Figure 5.1). Therefore, exponential smoothing was employed to forecast the demand for weeks 21 to 25. With an alpha value of 0.6, recent data was given more weight, ensuring a Mean Absolute Deviation (MAD) below 10 (Table 5.1).

# 3.1.1 Alternative Research Forecasting Methods
(a) Sarimax:

Sarimax, or Seasonal Autoregressive Integrated Moving Average, is a forecasting method geared towards analyzing the effects of demand-influencing factors to predict future values. It's particularly adept at handling data with seasonal fluctuations, employing backward stepwise regression to estimate parameters and address seasonality. Similar to exponential smoothing, Sarimax excels in short-term load forecasting and accommodates external variables. For instance, for product five, Sarimax utilized an exponential smoothing model with an alpha value of 0.2. By combining exponential smoothing with SARIMAX, the model effectively captures short-term changes while accounting for external influences, resulting in more accurate predictions (Alharbi, 2022, p 94).

# (b) Back propagation Neural Networking:

Backpropagation Neural Networking, a form of time series forecasting, involves isolating neural networks within economics and finance. These networks consist of categorical layers with directional input and output, capturing temporal biases and weights. Unlike linear model networks, the inclusion of hidden layers distinguishes Backpropagation Neural Networking by capturing biases and inputs from multiple sources (Kaastra, 1996, p 215).

# (c) Trigonometric Seasonal Forecasting (TBATS):

TBATS, or Trigonometric Seasonal Forecasting, simplifies seasonal trends into single values over specific time frames. It utilizes trigonometric functions to accurately represent seasonal trends, making it suitable for datasets with regular, repeating seasonal patterns. By adjusting harmonic frequencies and smoothing parameters, TBATS determines the best trend model based on observed seasonal harmonization. For instance, for product three, TBATS effectively captures harmonic frequencies occurring every five months (Livera, 2011, p 1516).

# (d) Long Short-Term Memory Forecasting (LSTM-RNN):

LTSM, found in Recurrent Neural Networks, is widely used in finance, AI, and medicine for long-term data pattern recognition. LTSM excels at identifying repeated patterns within neural networks, disregarding time constraints to predict trends effectively. By analyzing data through different layers in the network, LTSM can predict outcomes without limitations, aiding in precise estimations without clear end dates (Vu, 2020, p 4).

# 3.2 Aggregate Planning

In developing an aggregate production plan for the next five weeks, our aim is to efficiently meet forecasted demand while minimizing total production costs. We will employ both level and chase production strategies for each product type, considering the available manufacturing resources and workforce.

For the level production strategy, we aim to maintain a constant production rate over the planning horizon, adjusting inventory levels to match demand fluctuations. This approach helps stabilize workforce levels and minimizes overtime or under time. Conversely, the chase production strategy involves adjusting production to match demand closely, minimizing inventory holding costs but potentially requiring hiring or layoffs to accommodate demand fluctuations. Using historical demand data and the forecasted demand, we will develop a production plan that balances inventory levels, backorders, and workforce utilization. By optimizing the allocation of resources and adjusting production levels as needed, we aim to achieve zero net inventory while minimizing idle time and labor costs. To implement the production plan, we will also develop Material Requirements Planning (MRP) records for all products and sub-products based on the provided Bill of Materials (BOM). This will help ensure that the necessary raw materials and components are available to meet production requirements.

Furthermore, we will create a Master Production Schedule (MPS) for the sub-products based on the MRP records. The MPS will outline the production schedule for each sub-product, aligning with forecasted demand and considering lead times for procurement and production.

In addition to developing the production plan and MPS, we will conduct a capacity plan to determine the number of machines required to support production activities. By considering machine capacity and downtime, we can ensure that production schedules are feasible and optimize resource utilization.

Overall, our aggregate production planning approach aims to optimize production efficiency, minimize costs, and meet customer demand effectively. By balancing production levels, inventory levels, and workforce utilization, we can achieve operational excellence and maximize profitability for Vikings Division.

# 3.3 Master Production Scheduling

The Master Production Schedule (MPS) serves as a vital tool for coordinating production activities and ensuring timely delivery while avoiding inventory excess. By aligning sales demand with production capacity, the MPS provides a framework for generating each product at the optimal times. It's a dynamic process, continually updated with real orders and the most recent data to reflect changes in demand, capacity constraints, and inventory levels. The MPS considers various factors such as forecasts, known orders, capacity restrictions, and inventory status to develop a feasible production schedule. Utilizing the Bill of Materials (BOM), we calculated the gross need for each sub-product. This involved considering the anticipated available balance, lot size, lead time, and safety stock to determine the material requirements. The Material Requirements Planning (MRP) process then determined when each sub-product should be produced to meet production demands while minimizing inventory costs. The projected order receipts for each sub-product were generated based on these calculations, aiming to optimize process efficiency and reduce inventory expenses. The Master Production Schedule (MPS) outlines the production quantities for individual products, guiding production activities according to demand forecasts. Each product has a unique MPS, detailing the weekly production quantity. This enables effective coordination between sales and production, ensuring seamless interaction and alignment of production activities with sales demand.

# 3.4 Materials Requirement Planning (MRP)

Material Requirements Planning (MRP) plays a crucial role in material planning and control, providing essential data for capacity planning and production execution. The MRP system utilizes information such as the bill of materials for sub-products and inventory status to generate specific instructions for production planning. Safety stock levels and lead times are taken into account when calculating MRPs for sub-components, ensuring adequate inventory levels to meet demand fluctuations. The MRP process involves determining the additional material required based on gross requirements and projected available balance quantities. In cases where demand exceeds available inventory, exceptions messages indicate the need to roll over requirements to preceding periods. For example, at Week 20, Sub-product 4's exception message indicates that the requirement has been carried over due to sufficient projected available balance from the previous period, ensuring production continuity without disruption.

# 3.5 Capacity Planning

Based on the provided information and Bill of Materials, a resource profile was established for the products and sub-products. This profile indicated that two level sub-products contribute to products two through five. With a lead time of one week for sub-products one, three, and four, the time required at each station to produce one unit of a product was determined. This data, along with forecasted demand for each product, was used to calculate the capacity at each station for each week, aggregated to determine the overall necessary capacity to meet forecasted demand.

An analysis was conducted to determine the optimal approach to acquiring the necessary capacity. The first analysis calculated the minimum number of machines required at each station to meet the highest capacity period, ensuring machines are purchased or sold at the first production period and retained throughout. The second analysis calculated the number of machines needed per period per station, allowing for machines to be purchased and sold each period based on the minimum required capacity.

The second analysis, involving purchasing and selling machines each period, emerged as the most cost-effective option. This approach offers flexibility to adjust capacity as demand fluctuates, minimizing costs associated with underutilized capacity. The highlighted costs from the analysis totaled approximately $372,848, allowing for adjustments based on demand fluctuations.

In contrast, the first analysis, involving a fixed number of machines for all periods, incurred higher costs totaling about $1,950,484. While this approach guarantees fixed capacity to accommodate unexpected orders, it may result in higher costs in periods of lower demand. Ultimately, the decision between maintaining a fixed number of machines or purchasing and selling machines each period depends on distributions anticipation of demand fluctuations and the need to accommodate unexpected orders. If higher demand or unexpected orders are anticipated, maintaining a fixed capacity may be preferable. However, if demand is expected to remain stable or if flexibility is desired, the option of purchasing and selling machines each period offers a more cost-effective solution.

# 3.6 Simulation and Scheduling

When designing the scheduling system using ProModel, the primary objective was to optimize production, minimize cycle time, and meet the production plan for the first week. The structure of the ProModel simulation was organized by locations, stations, and queues. Products and sub-products served as entities that moved through the system via stations and operations. Arrival entities, such as product 1, were routed through different stations based on predefined logic, while sub-products arrived at specific queues and were routed accordingly. Batch size and sub-product type were used to control the flow and behavior of the process. Changeover logic was implemented to manage transitions between sub-products at stations, with wait times accounted for.

Routing logic ensured that entities moved through the system efficiently, either proceeding to the next station or exiting the system upon completion. Analysis of the simulation results revealed that while station 1 experienced high entry counts, station 6 had relatively lower utilization, indicating a potential bottleneck. Product 4 exhibited faster processing times compared to others, while sub-products with higher counts demonstrated greater demand. Some stations exhibited low entry counts, suggesting bottleneck occurrences.

Subsequent adjustments were made to the simulation to alleviate bottlenecks, resulting in unintended consequences such as all product 5 and sub-products 1 and 2 experiencing bottlenecks, leading to zero total exits. Despite challenges, the scheduling methodology aimed to release an equal number of products, aligning with the initial capacity bills process. The ProModel simulation enabled optimization of the production process and reduction of cycle time.

To further analyze system behavior and identify areas for improvement, the simulation was set with a warm-up time of 8, runtime of 400, and 10 replications, incorporating variables and attributes for comprehensive analysis. This approach allows for deeper exploration of the system's behavior and informs potential adjustments to enhance workflow efficiency.

Top of Form

# 4. Results

Through the comprehensive analysis of our forecasting methods and aggregate planning strategies for each product, we have gained valuable insights into the most effective approaches to meet demand while optimizing costs.

For Product 1, we explored both exponential smoothing and exponential smoothing with trend methods. Despite slightly lower MAD values in the first forecast, we ultimately chose the latter method due to its cost-effectiveness demonstrated through aggregate planning. Both Chase and Level aggregate planning methods were utilized, with the Chase methodology proving more economical.

Product 2's seasonal trend guided our choice of exponential smoothing with seasonality and trend method for forecasting. Chase aggregate planning was recommended based on minimal cost differences compared to Level planning.

The upward trend observed in Product 3's demand led us to employ Seasonality with trend forecasting method. The chosen method yielded a MAD within the specified tolerance level, ensuring accurate forecasts.

Product 4's demand trend indicated the need for exponential smoothing with trend adjustment forecasting method. This method effectively captured demand fluctuations, resulting in low MAD values.

Product 5's forecasting involved exponential smoothing with varying alpha values to ensure accurate predictions. Despite some deviation, selecting an alpha of 0.2 met our criteria for successful forecasting, as validated by MAD values.

Through Materials Requirement Planning (MRP), we determined the necessary sub-products required for production, considering lot sizing, lead time, and safety stock. Capacity planning highlighted the costs associated with maintaining fixed capacity versus adjusting capacity based on demand fluctuations. The analysis revealed that flexible capacity adjustment yielded lower costs, making it a more cost-effective option.

Simulation scheduling using ProModel provided insights into production flow and identified potential bottlenecks. Adjustments were made to optimize production efficiency, ensuring a consistent flow of products through the system.

Overall, our project outcomes underscore the importance of selecting appropriate forecasting methods, aggregate planning strategies, and production optimization techniques to meet demand efficiently while minimizing costs.

# 5. Conclusions and Recommendations

The team devised a cost-saving plan aimed at bolstering Vikings Division's competitiveness in the nutritional products market while meeting customer demands effectively. With the assistance of Mr. Smith and the insights provided in this comprehensive report, we anticipate Vikings Division's resurgence in the market. Our recommendations, outlined in Table 1, offer a clear path to cost reduction and improved efficiency. The aggregate planning analysis presented in Figure 33 highlights the optimal cost-saving measures identified through our thorough evaluation process.

# 6. References

# Tarsitano,A.,&Amerise,I.L.(2017).Short-termloadforecastingusingatwo-stagesarimaxmodel.

# Energy(Oxford),133,108–114. https://doi.org/10.1016/j.energy.2017.05.126

# Alharbi, F. R., &Csala, D. (2022). A Seasonal Autoregressive Integrated Moving Average with Exogenous Factors (SARIMAX) Forecasting Model-Based Time Series Approach. Inventions (Basel), 7(4), 94–. https://doi.org/10.3390/inventions7040094

# Vu,T.,Vu,P.,PubliéparThanhVuVoirsesArticles,&*,N.(2020,October13).LongShort-Term Memory (LSTM) networks for time series forecasting. Publicis Sapient Engineering - Engineering Done Right. https://blog.engineering.publicissapient.fr/2020/09/23/long-short- term-memory-lstm-networks-for-time-series-forecasting/

# Kaastra,I.,&Boyd,M.S.(1996).Designinganeuralnetworkforforecastingfinancialandeconomic time series. Neurocomputing, No. 10, pp. 215-236.

# Alysha M. De Livera, Rob J. Hyndman and Ralph D. Snyder. (December 2011). Journal of the American Statistical Association. Vol. 106, No. 496, pp. 1513-1527

# 7. Appendix

# Table 1. Historical Demand

# Table 2. Bill of Materials

Product 2

Sub-Product 4

Sub-Product 3

(2 required)

Sub-Product 4

(4 required)

Product 1

Sub-Product 4

Sub-Product 1
(3 required)

Product 3

Sub-Product 1

Sub-Product 3

Sub-Product 4

(4 required)

Product 4

Sub-Product 4

Sub-Product 2

Sub-Product 1

(5 required)

Product 5

Sub-Product 1

Sub-Product 2

Sub-Product 1

(5 required)

# Table 3(a). Cost Information of Products

|  | Product 1 | Product 2 | Product 3 | Product 4 | Product 5 |
| --- | --- | --- | --- | --- | --- |
| Regular Cost ($/unit) | 100 | 80 | 25 | 150 | 200 |
| Holding ($/unit/week) | 15 | 12 | 11.5 | 20 | 18.5 |
| Backorder ($/unit/week) | 25 | 18 | 10 | 50 | 75 |

|  | Product 1 | Product 2 | Product 3 | Product 4 | Product 5 |
| --- | --- | --- | --- | --- | --- |
| Regular Cost ($/unit) | 100 | 80 | 25 | 150 | 200 |
| Holding ($/unit/week) | 15 | 12 | 11.5 | 20 | 18.5 |
| Backorder ($/unit/week) | 25 | 18 | 10 | 50 | 75 |

# Table 3(b) Cost Information of Sub-Products

|  | Sub-Product 1 | Sub-Product 2 | Sub-Product 3 | Sub-Product 4 |
| --- | --- | --- | --- | --- |
| Regular Cost ($/unit) | 7 | 25 | 22 | 12 |

## Table 4. Routings

## Table 5. Number of Machines

| Station | Number of Machines |
| --- | --- |
| 1 | 10 |
| 2 | 20 |
| 3 | 10 |
| 4 | 15 |
| 5 | 15 |
| 6 | 20 |
| 7 | 5 |

## Table 6. Processing Times (minutes per unit)

| Station | Sub-Product 1 | Sub-Product 2 | Sub-Product 3 | Sub-Product 4 |
| --- | --- | --- | --- | --- |
| 1 | 20 | 19 | 13 | 0 |
| 2 | 9 | 22 | 18 | 8 |
| 3 | 14 | 18 | 21 | 0 |
| 4 | 21 | 7 | 0 | 12 |
| 5 | 15 | 15 | 16 | 16 |
| 6 | 11 | 11 | 0 | 15 |
| 7 | 18 | 0 | 3 | 10 |

## Table 7. Setup Times (minutes per lot)

| Station | Sub-Product 1 | Sub-Product 2 | Sub-Product 3 | Sub-Product 4 |
| --- | --- | --- | --- | --- |
| 1 | 200 | 190 | 80 | 0 |
| 2 | 130 | 215 | 160 | 130 |
| 3 | 210 | 85 | 65 | 0 |
| 4 | 95 | 155 | 0 | 60 |
| 5 | 70 | 125 | 120 | 75 |
| 6 | 110 | 75 | 0 | 125 |
| 7 | 150 | 0 | 125 | 270 |

# Table 8. Downtime (minutes per machine)